## Asphalt Shingle Roof Inspection Protocol

Company Name: [Insert Company Name]
Inspector: [Insert Inspector Name]
Date of Inspection: [Insert Date]
Property Address: [Insert Building Name or Address]
Roof Section(s): [Insert Area or Elevation ID if needed]

### I. PRE-INSPECTION PREPARATION

- Document Review:

  - Gather installation records, warranty, and previous repair history.

  - Confirm roof age, shingle type (3-tab, architectural, impact-resistant), and number of layers.

- Weather & Safety Verification:

  - Schedule inspection during dry weather.

  - Confirm access safety: fall protection, ladders, PPE.

  - Notify property occupants if required.

### II. GROUND-LEVEL OBSERVATION

- General Visual Inspection:

  - Check for shingle sagging, uneven lines, or visible dips.

  - Look for signs of previous repairs or shingle patches.

  - Examine eaves for visible granule loss in gutters/downspouts.

- Soffit, Fascia & Gutter Assessment:

  - Look for water stains, rot, rust, or detachment.

  - Inspect gutter slope, fasteners, and debris accumulation.

### III. ROOF SURFACE INSPECTION (ON-ROOF)

- Shingle Field Assessment:

  - Look for missing, torn, cracked, or curling shingles.

  - Identify signs of granule loss, blistering, or algae streaks.

  - Check for thermal cracking or cupping due to aging.

- Nail and Fastener Check:

  - Inspect for exposed nails, nail pops, and lifted tabs.

  - Note under-driven or over-driven fasteners.

- Flashing & Transition Details:

  - Inspect step flashings at walls, headwalls, and chimneys.

  - Look for lifted or corroded flashing, missing sealant, or improper overlaps.

  - Examine counterflashing embedment and watertightness.

- Valleys:

  - Check for debris, damage, or signs of granule washout.

  - Verify installation type (open metal valley, closed cut, woven).

  - Look for cracks or breaks in valley shingles or metal.

- Penetrations:

  - Inspect pipe boots, vents, attic fans, and satellite mounts.

  - Check for cracked gaskets, unsealed fasteners, or deteriorated flashing.

  - Ensure storm collars and sealants are intact.

- Ridges and Hips:

  - Evaluate ridge cap shingle condition for brittleness or cracks.

  - Ensure ridge vent is intact and properly fastened.

### IV. ATTIC & INTERIOR ROOF INSPECTION

- Attic Conditions (if accessible):

  - Check for signs of leaks, water stains, mold, or rot.

  - Inspect roof deck for deflection, rot, or delamination.

  - Verify insulation coverage and look for daylight at roof penetrations.

- Ventilation Assessment:

  - Check for balanced intake and exhaust (soffit, ridge, gable, etc.).

  - Look for evidence of condensation or inadequate airflow.

### V. STORM DAMAGE CHECKPOINTS

- Hail Damage:

  - Identify circular bruising or granule displacement on shingles.

  - Verify mat fracture by touch or minor lift test.

- Wind Damage:

  - Look for lifted, creased, or missing shingles (especially at edges and ridges).

  - Assess seal strip integrity and fastener uplift resistance.

- Ice Damming (in cold climates):

  - Identify previous ice dam water stains near eaves or soffits.

  - Inspect roof edge insulation and ventilation adequacy.

### VI. DOCUMENTATION & REPORTING

- Photographic Records:

  - Capture high-resolution images of all noted deficiencies and overall roof condition.

  - Include marked images showing roof slope, elevations, and penetration areas.

- Annotated Roof Diagram:

  - Include sketch or aerial image with markings showing defect locations.

- Condition Rating System:

  - ✅ Good – Minimal wear, no active issues

  - ⚠️ Fair – Aging signs, repair recommended

  - ❌ Poor – Active leaks, multiple failures, nearing replacement

- Repair Recommendations:

  - Immediate (1–7 days): Leaks, missing shingles, flashing gaps

  - Short-Term (1–3 months): Nail pops, minor granule loss

  - Long-Term (6–12 months): Aging roof, full replacement planning

### VII. MAINTENANCE & FOLLOW-UP PLAN

- Recommend seasonal (spring/fall) roof checkups.

- Clean gutters and remove roof debris regularly.

- Document post-storm conditions and inspect after major wind or hail events.

Inspector Signature: __________________________
Client/Owner Signature: ________________________
Date of Report Completion: _____________________